System-level resilience may mask substantial differences in the timing and location of service restoration. Building on the simulator outputs used in previous works, this task introduces equity-oriented measures and a Community Resilience Index (CRI) to quantify not only aggregate recovery, but also the distribution of recovery outcomes across the study area.

To make distributional outcomes explicit, the study area will be partitioned into analysis zones consistent with the demand and load mappings. For each zone z and time t, zone-level quantities will be tracked, including (i) electric service availability  and (ii) a transportation accessibility indicator aligned with the recovery setting (e.g., zone access to specified essential destinations under prevailing road and power states).

These zone-level quantities will be combined into a zone-level CRI, denoted CRIz(t), using clearly defined normalization and weights. In this construction, the zone-level indicators are the CRI components, and equity is evaluated through how CRIz(t)(and associated restoration-time summaries) varies across zones. Some alternative weight/threshold choices may be examined through sensitivity analysis to avoid over-interpreting any single specification.

Based on CRIz(t), Task B will define equity measures that reflect the distribution of recovery outcomes. Candidate measures include dispersion-type metrics across zones (e.g., variance- or Gini-like indices of restoration duration), percentile-based targets (e.g., time by which a large share of zones or customers reach an adequate service level), and critical access status measures when essential facilities are included in the accessibility component. These measures are intended to support both reporting and the design of equity-oriented restoration rules.

Task B will then examine efficiency–equity trade-offs without assuming that a single “optimal” formulation is available or necessary. The approach will generate a set of feasible repair sequences by systematically varying how equity is incorporated into decision rules as well as arbitrary tuning weights/parameters, while evaluating every sequence on both (i) resilience-triangle performance and (ii) equity/CRI outcomes. For example, equity can be introduced by reweighting toward lower-service zones, or simple guardrail constraints that limit excessive disparities. The analysis can then produce an empirical trade-off curve and identify sequences that are non-dominated with respect to the two objectives.

Trade-offs will be summarized by comparing changes in resilience-triangle loss against changes in equity indicators across this parameterized set of sequences. This task supports RQ2 by quantifying how prioritization choices influence both overall recovery efficiency and the distribution of recovery benefits, within the same interdependent recovery setting and scenario specifications used before.